Textiles — Guideline for banned or restricted substances

Table of contents

1	Scope	1

2	Normative references	1

3	Restricted list	1

Annex A	6

Banned Aromatic Amines Specified in EC 2002/61	6

Annex B	7

Examples of dyes which potentially cleave to form BANNED Aromatic Amines (listed in Annex A) under reducing conditions	7

Annex C	11

BANNED Dyestuffs implicated in Contact Dermatitis by Consumers, which include:	11

Annex D	12

Pesticides / Insecticides include the following:	12

Annex E	14

#### Foreword

The African Organization for Standardization (ARSO) is an African intergovernmental organization established by the United Nations Economic Commission for Africa (UNECA) and the Organization of African Unity (AU) in 1977. One of the fundamental mandates of ARSO is to develop and harmonize African Standards (ARS) for the purpose of enhancing Africa’s internal trading capacity, increase Africa’s product and service competitiveness globally and uplift the welfare of African communities. The work of preparing African Standards is normally carried out through ARSO technical committees. Each Member State interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, Regional Economic Communities (RECs), governmental and non-governmental organizations, in liaison with ARSO, also take part in the work.

ARSO Standards are drafted in accordance with the rules given in the ISO/IEC Directives, Part 2.

The main task of technical committees is to prepare ARSO Standards. Draft ARSO Standards adopted by the technical committees are circulated to the member bodies for voting. Publication as an ARSO Standard requires approval by at least 75 % of the member bodies casting a vote.

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ARSO shall not be held responsible for identifying any or all such patent rights.

This African Standard was prepared by ARSO TC 053, Textiles, textile products and accessories.

© African Organisation for Standardisation 2026 — All rights reserved*

ARSO Central Secretariat

International House 3rd Floor

P. O. Box 57363 — 00200 City Square

NAIROBI, KENYA

Tel. +254-20-2224561, +254-20-3311641, +254-20-3311608

E-mail: arso@arso-oran.org

Web: www.arso-oran.org

Copyright notice This ARSO document is copyright-protected by ARSO. While the reproduction of this document by participants in the ARSO standards development process is permitted without prior permission from ARSO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ARSO. Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ARSO’s member body in the country of the requester: © African Organisation for Standardisation 2026 — All rights reserved ARSO Central Secretariat International House 3rd Floor P.O. Box 57363 — 00200 City Square NAIROBI, KENYA Tel: +254-20-2224561, +254-20-3311641, +254-20-3311608 E-mail: arso@arso-oran.org Web: www.arso-oran.org Reproduction for sales purposes may be subject to royalty payments or a licensing agreement. Violators may be prosecuted.

AFRICAN STANDARD

Textiles — Guidelines for banned or restricted substances

# 1	Scope

This Draft African Standard provides a guideline for some of the common substances that are banned or restricted within certain limits for use in textiles and apparel.

Note

This is only a guideline, and legal legislation precedes this document. Compliance with this document cannot confer immunity from legal obligations. The list stated in this document is not conclusive of all banned or restricted substances

# 2	Normative references

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ARS 1551, Textiles and textile merchandise — Terms and definitions

ISO 24047, Plastics - Polyethylene (PE) and polypropylene (PP) thermoplastics - Determination of metal content by ICP-OES

ISO 14184-2 Textiles - Determination of formaldehyde Part 2: Released formaldehyde (vapour absorption method)

ISO 17070 Leather-Chemical tests- Determination of tetrachlorophenol-, trichlorophenol-, dichlorophenol-, monochlorophenol-isomers and pentachlorophenol content

ISO 14362-1, Textiles-Methods for determination of certain aromatic amines derived from azo colorants

Part 1: Detection of the use of certain azo colorants accessible with and without extracting the fibres

DIN 54231:2022-09, Textiles - Determination of dyes after methanol extraction

ISO 14184-1 Textiles-Determination of formaldehyde Part 1: Free and hydrolysed formaldehyde (water extraction method)

ISO 17075-1, | IULTCS/IUC 18-1, Leather — Chemical determination of chromium(VI) content in leather Part 1: Colorimetric method

ISO 17226-1 Leather — Chemical determination of formaldehyde content — Part 1: Method using high-performance liquid chromatography

ISO 17226-2 | IULTCS/IUC 19-2, Leather — Chemical determination of formaldehyde content

Part 2: Method using colorimetric analysis

ISO/TS 17234, Leather — Chemical tests — Determination of certain azo colourants in dyed leathers

ISO 17353, Water quality — Determination of selected organotin compounds — Gas chromatographic method

SATRA TM277, Determination of organotins

# 3	Terms and definitions

For the purposes of this document, the terms and definitions given in ARS 1551 apply.

# 4 Requirements

## 4.1 Restricted list

4.1.1	There are many thousands of chemicals that are known to be harmful to humans or the environment. The restricted and banned chemicals, dyes and substances are listed in Table 1 and Annex A to E. The list does not cover all banned or restricted substances for textiles and apparels.

4.1.2 Import and export of textiles and apparels shall comply with the relevant country’s (destination of goods) regulations for banned and restricted substances.

4.1.3	The following additional restrictions apply to children’s wear apparel:

a)	formaldehyde – deliberate use of Formaldehyde containing products not permitted in underwear or babywear.

b)	PVC and Phthalates – not permitted.

c)	biocidal finishes – not permitted unless by consent; and

d)	flame retardant finishes – not permitted.

Table 1 Restricted or banned chemicals

| CHEMICAL | ORIGIN | Why it is banned or restricted? | Maximum limit on a finished product | Practical limit of detection | Test method |
| --- | --- | --- | --- | --- | --- |
| Banned Azo dyes and pigments No deliberate use | Banned amines listed in annex A Examples of dyes that can form such amines listed in annex B | Dyes can split to form carcinogenic amines | 20ppm (mg/kg). | 5ppm (mg/kg) | Textiles ISO 14362-1 Leather ISO/TS 17234 |
| WARNING: Azo pigments - CI Pigment Orange 13 and CI Pigment Orange 34, when used in combination with CI Pigment Black 7, can release the banned aromatic amine 3,3’ – dichlorobenzidine. Do not use these pigments in combination. |  |  |  |  |  |
| Skin Sensitising Disperse Dyes No deliberate use | Disperse Dyes listed in Annex C | Once sensitised to a dye, people can react violently to trace quantities | 20 ppm | 5 ppm | DIN 54231:2022-09 |
| Alkyl phenol ethoxylates (APEO's, including NPEO and OPEO) No deliberate use | In widespread use as detergents, Wetting agents, and as emulsifying agents | Endocrine disruptors (sex change chemicals) for aquatic species | 500 ppm combined total of NPEO and OPEO | 100 ppm | Solvent Extraction |
| Organo tin compounds No deliberate use | Preservative for fabrics and chemical formulations. Occasionally used as stabilizers and catalysts | Tributyl tin is highly toxic, and related products have toxicity issues | 0.5 ppm TBT, TPT (in extract) 1ppm DBT, DOT and MBT (in extract) | 0.05 ppm | Textiles: Solvent extraction ISO 17353 Leather: SATRA TM277 |
| Chlorinated Phenols Including PCP and derivatives, TeCP No deliberate use | Preservative for cotton and viscose. Main risk is on imported greige fabrics | Highly Toxic | 1ppm total of PCP plus TeCP Leather 0.5ppm | 0.05ppm | ISO 17070 |
| Mercury No deliberate use. | Caustic soda that is made by the 'mercury cell process'. | Highly toxic. | 0.02 ppm. | 0.01 ppb. | Any appropriate technique e.g. Combustion amalgamation with cold vapour detection. |
| Mothproofing: Permethrin & Analogues of these No deliberate use. | Common chemical is permethrin - found on wool and cashmere. | Nerve agent, and toxic to aquatic species. | Contamination limit 10ppm. | 1 ppm. | Solvent extraction. |
| PFOS No deliberate use. | Water-repellent ‘8-chain’ fluorocarbon finishes based on electrofluorination (old generation products). | Proven health risks, and persistent in the environment. | 1 μg/m2. | 1 ppm. | Solvent extraction. |
| PFOA No deliberate use. | Water-repellent finishes based on telemerisation. | Persistent in the environment and suspected health risks. | 1 μg/m2. | 1ppm. | Solvent extraction. |
| Dye Carriers Including Chlorobenzenes, BPP and OPP No deliberate use. | Used to dye polyester and blends at low temperatures in machinery not capable of being pressurised. | Various depending on type of carrier - generally toxic, irritants or carcinogens. | 1 ppm total for halogenated aromatic hydrocarbons, biphenyl or ortho phenyl phenol. | 1 ppm. | Solvent extraction. |
| Chromium VI Consent Required. | Chromium compounds used in 2-stage 'after-chrome' wool dyeing. | Highly toxic / carcinogenic both to humans and aquatic species. | Textiles and Leather 3ppm. | 3 ppm. | ISO 17075-1. |
| Organic solvents Including chlorinated solvents Consent Required. | Organic solvents listed in Annex E. Used in some adhesives, print formulations, solvent scouring and spot cleaning. | Various depending on type of solvent. | Limit of detection 1 ppm. | Varies according to chemical type. | Solvent extraction. |
| Organic solvent technical notes - Where solvents are used suppliers should always work to change to water based alternatives. Where this is not possible e.g. dry pigment discharge printing there must be adequate extraction of fumes, good ventilation, and workers must be provided with appropriate Personal Protective Equipment. Solvents are permitted for scouring greige, but these must be in fully enclosed zero emission systems. No residual solvent is permitted on any finished product. Trichloroethylene is classified as a carcinogen.Do not use the product if it has the CAS number 79-01-6.. |
| Biocidal Finishes Consent Required Not permitted in Childrenswear unless by consent Triclosan is not permitted | Deliberate application on fabrics | Toxic Can cause severe skin irritation e.g. DMFu (see below) | Limit of detection unless agreed in writing by technologist | Varies according to type. Report result as ppm | Analytical – Solvent extraction |
| Biocidal finish notes Deliberately applied biocidal finishes to impart customer benefit must be permanent, non-leaching, and work only on the fabric and not on customers’ skin, and must not be implicated in antibiotic resistance. Biocidal finishes should not be confused with odour absorbers such as cyclodextrins. |  |  |  |  |  |
| Flame retardants Consent Required Not permitted in Childrenswear | Deliberate application on fabrics | Depends on exact chemistry - Toxic, not biodegradable and suspected health risks | 5ppm for penta-, hexa- and octa -brominated biphenyl ethers, PCB’s, PCT’s HBCDD, TCPP, TCEP and TDCPP | 5 ppm | Solvent extraction |
| Flame retardant notes Flame retardant finishes should only be applied where there is a legislative need with written consent. Penta, Hexa and Octa –brominated types must not be deliberately applied and must not be present above 5 ppm. |  |  |  |  |  |
| Phthalates (as softener for eg PVC) Not permitted in Children’s clothing | Used to soften rigid PVC | Suspected sex change chemicals /suspected carcinogen | 1000 ppm for accessories 500 ppm for textiles | 100 ppm | Solvent extraction |
| Phthalate and PVC notes It is essential that all suppliers who are using PVC based products familiarise themselves with, and comply with the PVC policy. DEHP, DBP and BBP are SVHCs, and are banned above 1000ppm in all childcare articles and toys. DINP, DIDP and DOP are banned above 1000ppm in articles that are intended for children |  |  |  |  |  |
| Formaldehyde | Most commonly found in resinated products (and in lower levels in fixing agents for cotton and nylon) | Known irritant to skin and mucous membranes Reclassified as carcinogen by WHO | Skin-contact adult wear ≤ 75 ppm. Non-skin contact adult wear ≤ 300 ppm. 20 ppm for baby wear – not direct skin contact Not detected for baby wear - direct skin contact eg. underwear | Less than 20 ppm | Textiles: ISO 14184 Parts 1 and 2. Leather: ISO 17226-1 & ISO 17226-2 |
| Pesticides / Insecticides No deliberate use | Pesticides and Insecticides listed in Annex D. Used to protect sheep and cotton crops from parasite infestation. | Toxic | Total 0.5ppm for wool Total 0.05ppm for cotton | 0.05 ppm | Analytical |
| pH of Textiles and Leather | Acidity or alkalinity arising from the final process | Extreme values can cause skin irritation | direct skin contact pH 4.0 – 7.5 No direct skin contact pH 4.0 – 8.5 |  |  |
| Metals in Textiles - Technical Notes There is specific legislation relating to specific metals in all end uses (e.g. Cadmium Directive) and also legislation for metals in toys (EN71). (see also Nickel Policy) |  |  |  |  |  |
| Cadmium | Pigments, Dyes, Fibres, Alloys | Toxic | 0.1 ppm | 0.03 ppm | ISO 24047 |
| Nickel | Green and turquoise dyes | Allergenic | 4 ppm | 1 ppm | ISO 24047 |
| Antimony | Fibres, flame retardants | Toxic | 30 ppm | 30 ppm | ISO 24047 |
| Lead | Pigments / water pipes | Toxic | 1 ppm | 0.3 ppm | ISO 24047 |
| Copper | - | Toxic | Adults: 50ppm Infants: 25ppm | 0.3 ppm | - |
| Chromium | - | Toxic | Adults: 2 ppm Infants: 1 ppm | 0.3 ppm | - |
| Other Heavy Metals | Various | Toxic | 1ppm | 0.3 ppm | ISO 24047 |
| Chlorine based bleach Sodium Chlorite is not permitted | Deliberate application | Toxic in high concentrations. Concerns over AOX formation in effluent | Use chlorine-free bleach where practicable | N/A | - |
| Chlorine Technical notes The use of alternative bleaching systems based on, eg hydrogen peroxide are recommended. The use of sodium chlorite in particular is prohibited. |  |  |  |  |  |

# Annex A

# (Normative)

# Banned Aromatic Amines Specified in EC 2002/61

| TEST METHOD ISO 14362-1 USED TO DETECT BANNED AMINES (1-21 ON THIS LIST). If aniline, or 1,4 – phenylenediamine is detected, see amine number 22 (below) for special test method |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | CAS Number | Index Number | EC Number | Substances |
| 1 | 92-67-1 | 612-072-00-6 | 202-177-1 | 4-aminobiphenyl (4-ABP) biphenyl-4-ylamine 4-aminobiphenyl xenylamine |
| 2 | 92-87-5 | 612-042-00-2 | 202-199-1 | Benzidine |
| 3 | 95-69-2 | 612-196-00-0 | 202-441-6 | 4-chloro-o-toluidine |
| 4 | 91-59-8 | 612-022-00-3 | 202-080-4 | 2-naphthylamine |
| 5 | 97-56-3 | 611-006-00-3 | 202-591-2 | o-aminoazotoluene 4-amino-2',3-dimethylazobenzene 4-o-tolylazo-o-toluidine |
| 6 | 99-55-8 | 612-210-00-5 | 202-765-8 | 5-nitro-o-toluidine |
| 7 | 106-47-8 | 612-137-00-9 | 203-401-0 | 4-chloroaniline |
| 8 | 615-05-4 | 612-200-00-0 | 210-406-1 | 4-methoxy-m-phenylenediamine |
| 9 | 101-77-9 | 612-051-00-1 | 202-974-4 | 4,4'-methylenedianiline 4,4'-diaminodiphenylmethane |
| 10 | 91-94-1 | 612-068-00-4 | 202-109-0 | 3,3’-dichlorobenzidine 3,3'-dichlorobiphenyl-4,4'-ylenedia mine |
| 11 | 119-90-4 | 612-036-00-X | 204-355-4 | 3,3’-dimethoxybenzidine |
| 12 | 119-93-7 | 612-041-00-7 | 204-358-0 | 3,3’-dimethylbenzidine |
| 13 | 838-88-0 | 612-085-00-7 | 212-658-8 | 4,4’-methylenedi-o-toluidine |
| 14 | 120-71-8 | 612-209-00-X | 204-419-1 | 6-methoxy-m-toluidine p-cresidine |
| 15 | 101-14-4 | 612-078-00-9 | 202-918-9 | 4,4’-methylene-bis-(2-chloro-aniline) |
| 16 | 101-80-4 | 612-199-00-7 | 202-977-0 | 4,4’-oxydianiline |
| 17 | 139-65-1 | 612-198-00-1 | 205-370-9 | 4,4’-thiodianiline |
| 18 | 95-53-4 | 612-091-00-X | 202-429-0 | o-toluidine 2-aminotoluene |
| 19 | 95-80-7 | 612-099-00-3 | 202-453-1 | 4-methyl-m-phenylenediamine |
| 20 | 137-17-7 | 612-197-00-6 | 205-282-0 | 2,4,5-trimethylaniline |
| 21 | 90-04-0 | 612-035-00-4 | 201-963-1 | o-anisidine 2-methox aniline |
| 22 | 60-09-3 | 611-008-00-4 | 200-453-6 | p-aminoazobenzene 4-amino azobenzene’ German Test No. 64 LFGB B82.02-9 September 2006 may be used to detect presence of Amine 22. If 4-amino azobenzine (para amino benzene) is detected: <20PPM = PASS, >20PPM = FAIL |
| 23 | 95-68-1 | - | 202-440-0 | 2,4-xylidine |
| 24 | 87-62-1 | 612-161-00-X | 201-758-7 | 2,6-xylidine |

# Annex B

# (Normative)

# Examples of dyes which potentially cleave to form BANNED Aromatic Amines (listed in Annex A) under reducing conditions

- CI Acid Orange 45 22195

- Direct Red 21 23560

- Direct Blue 173

- CI Acid Red 24 16140

- Direct Red 22 23565

- Direct Blue 192

- CI Acid Red 85 22245

- Direct Red 28 22120

- Direct Blue 201

- CI Acid Red 114 23635

- Direct Red 37 22240

- Direct Blue 215 24115

- CI Acid Red 115 27200

- Direct Red 39 23630

- Direct Blue 295 23820

- CI Acid Red 128 24125

- Direct Red 44 22500

- Direct Green 130280

- CI Acid Red 148 26665

- Direct Red 46 23050

- Direct Green 6 30295

- CI Acid Red 158 20530

- Direct Red 62 29175

- Direct Green 8 30315

- CI Acid Red 167

- Direct Red 67 23505

- Direct Green 8.1

- CI Acid Red 265 18129

- Direct Red 72 29200

- Direct Green 85 30387

- CI Acid Black 29

- Direct Violet 1 22570

- Direct Brown 1 30045

- CI Acid Black 209

- Direct Violet 12 22550

- Direct Brown 1:2 30110

- Azoic Diazo Component 12 37105

- Direct Violet 21 23520

- Direct Brown 2 22311

- Basic Brown 4 (= Solvent Brown 12) 21010

- Direct Violet 22 22480

- Direct Brown 6 30140

- Developer 14 (=Oxidation Base 20) 76035

- Direct Blue 1 24410

- Direct Brown 25 36030

- Direct Yellow 1 22250

- Direct Blue 2 22590

- Direct Brown 27 31725

- Direct Yellow 24 22010

- Direct Blue 3 23705

- Direct Brown 31 35660

- Direct Yellow 48 23660

- Direct Blue 6 22610

- Direct Brown 33 35520

- Direct Orange 1 22370

- Direct Blue 8 24140

- Direct Brown 51 31710

- Direct Orange 6 23375

- Direct Blue 9 24155

- Direct Brown 59 22345

- Direct Orange 7 23380

- Direct Blue 10 24340

- Direct Brown 79 30056

- Direct Orange 8 22130

- Direct Blue 14 23850

- Direct Brown 95 30145

- Direct Orange 10 23370

- Direct Blue 15 23790

- Direct Brown 101 31740

- Direct Orange 108 29173

- Direct Blue 22 24280

- Direct Brown 154 30120

- Direct Red 1 22310

- Direct Blue 25 23790

- Direct Brown 222 30368

- Direct Red 2 23500

- Direct Blue 35 24145

- Direct Black 4 30245

- Direct Red 7 24100

- Direct Blue 76 24411

- Direct Black 29 22580

- Direct Red 10 22145

- Direct Blue 151 24175

- Direct Black 38 30235

- Direct Red 13 22155

- Direct Blue 160

- Direct Black 154

- Direct Red 17 22150

# Annex C

# (Normative)

# BANNED Dyestuffs implicated in Contact Dermatitis by Consumers, which include:

- CI, Disperse Blue, 1

- CI, Disperse Blue, 3

- CI, Disperse Blue, 7

- CI, Disperse Blue, 26

- CI, Disperse Blue, 35

- CI, Disperse Blue, 102

- CI, Disperse Blue, 106

- CI, Disperse Blue, 124

- CI, Disperse Yellow, 1

- CI, Disperse Yellow, 3

- CI, Disperse Yellow, 9

- Cl, Disperse Yellow, 23

- CI, Disperse Yellow, 39

- CI, Disperse Yellow, 49

- CI, Disperse Orange, 1

- CI, Disperse Orange, 3

- CI,Disperse Orange,37

- CI,Disperse Orange,59

- CI,Disperse Orange,76

- CI, Disperse Red, 1

- CI, Disperse Red, 11

- CI, Disperse Red, 17

- CI, Disperse Brown, 1

# Annex D

# (Normative)

# Pesticides / Insecticides include the following:

D.1 Cotton and Natural Cellulosic Fabrics & Blends of these with Other Fibres Max Limits for each Product Listed 0.05ppm - Aldrin - Captafol - Chlordane - DDT - Dieldrin - Endrin - Heptachlor - Hexachlorobenzene - Hexachlorocyclohexane (total Isomers) - 2,4,5-T(richlorophenoxyacetic acid) - 2,4-D(ichlorophenoxyacetic acid) - Chlordimeform - Chlorobenzilate - Dinoseb (and its salts) - Monocrotophos - Cypermethrin - Deltamethrin - Fenvalerate

D.2 Wool and Other Keratin Fabrics & Blends of these with Other Fibres The Sum Total should Not Exceed 0.5ppm - Alpha-Hexachlorocyclohexane - Beta- Hexachlorocyclohexane - Gamma- Hexachlorocyclohexane - Lindane

- Aldrin

- Endrin

- Dieldrin

- DDT

- DDD

- Propetamphos

- Diazinon

- Dichlofenthion

- Fenchlorphos

- Chlorfenvionphos

- Cyhalothrin

# Annex E

# (Normative)

# Organic solvents

E.1 Depending on their type, organic solvents may be legally restricted e.g as carcinogens. No residual solvent is permitted on any finished product.

E.2 Organic solvents include the following but is not limited to:

Solvent

- Benzene

- Phenol

- Cresol (and isomers)

- Toluene

- Xylene (and isomers)

- Dimethyl formamide (DMF)

- Methyl ethyl ketone

- Bromodichloromethane

- Bromoform

- Tetrachloromethane (Carbon tetrachloride)

- Chlorodibromemethane

- Chloroethane

- Chloroform

- Dibromomethane

- Dichloroethane

- Dichloroethane

- Dichloroethene

- Dichloroethene

- Dichloropropene

- DichloromethaneTetrachloroethane

- Trichloroethane

- Tetrachloroethene (Tetrachloroethylene)

- Trichloroethene (Trichloroethylene)

- Vinyl chloride

- Hexachloroethane

Bibliography

Directive 2002/61/EC of the European Parliament

* 2026 ARSO — All rights of exploitation reserved worldwide for African Member States’ NSBs.